Emberwing

A dragon guarding its tower. The jaw opens. So does the treasure chest.

152 pieces  |  About 6 to 8 h  |  Ages 14+  |  No glue needed

Before you start

|  | Push parts out from the back of the sheet. Do not twist them out. |
| --- | --- |
|  | Every part has a code. B12 is part 12 on sheet B. Codes for show faces are printed next to the part. |
|  | Press tabs straight in, firm but gentle. No glue needed. |
|  | Too tight? Rub the tab on the sandpaper two or three times, then try again. |
|  | Rub wax into every hole that turns. It makes moving parts smooth. |
|  | A check mark means: test it now, before you close it up. |
|  | Small parts. Not for children under 14. Keep spare parts until you finish. |

What's in the box

Sheet A: Natural basswood, 38 parts

Sheet B: Natural basswood, 61 parts + 4 spares

Sheet C: Natural basswood, 24 parts + 2 spares

Sheet D: Red, 5 parts

Sheet E: Natural basswood, 4 parts

Sheet F: Gold, 17 parts

Sheet G: Purple, 2 parts

Sheet H: Green, 1 parts

1 x 3/16 in birch dowel, 21 mm (jaw)

1 x 3/16 in birch dowel, 56 mm (chest hinge)

1 x sandpaper 240 grit

Parts maps

| Step 1 Start the dragon: centre slice and keys |  |
| --- | --- |

| A11 A16 A20 A22 B6 B9 B14 B23 B28 C6 C20 D1 D3 |  |
| --- | --- |

The red centre slice carries the spikes. Push every key halfway through it.

| Step 2 Right side: slices 2 to 4 from the centre |  |
| --- | --- |

| B18 C2 E4 |  |
| --- | --- |

Slide each slice onto the keys. Match the outline to the slice below.

| Step 3 Right side: slices 5 to 7 from the centre |  |
| --- | --- |

| B7 B10 C3 C4 |  |
| --- | --- |

Slide each slice onto the keys. Match the outline to the slice below.

| Step 4 Right side: slices 8 to 10 from the centre |  |
| --- | --- |

| A7 A18 B17 B22 B29 B36 C19 |  |
| --- | --- |

Slide each slice onto the keys. Match the outline to the slice below.

| Step 5 Left side: slices 2 to 4 from the centre |  |
| --- | --- |

| B11 C5 C10 Dowel: 3/16 in dowel, 21 mm |  |
| --- | --- |

Same as the right side, mirror image.

| Step 6 Left side: slices 5 to 7 from the centre |  |
| --- | --- |

| B19 B26 B32 C9 |  |
| --- | --- |

Same as the right side, mirror image.

| Step 7 Left side: slices 8 to 10 from the centre |  |
| --- | --- |

| A3 B1 B3 B16 B21 B35 |  |
| --- | --- |

Same as the right side, mirror image.

| Step 8 Give her golden eyes |  |
| --- | --- |

| A2 x2 F1 F4 |  |
| --- | --- |

Each eye sits in its socket and is held by one square peg.

| Step 9 Attach the wings |  |
| --- | --- |

| A1 A10 B8 x2 B24 B30 F5 x2 G1 x2 |  |
| --- | --- |

Slide each purple wing forward through the shoulder bar. Then add the bones and the gold thumb claw with their pegs.

| Step 10 Lay the ground plate |  |
| --- | --- |

| A21 |  |
| --- | --- |

This is the rocky ground under the tower.

| Step 11 Build the tower walls |  |
| --- | --- |

| C1 E1 E2 E3 |  |
| --- | --- |

Front and back first, then the sides.

| Step 12 Put the top on the tower |  |
| --- | --- |

| A15 |  |
| --- | --- |

Tabs from all four walls lock it in.

| Step 13 Stand the battlements |  |
| --- | --- |

| B13 C7 C11 C12 x2 C13 C15 x3 C16 |  |
| --- | --- |

Broken ones are on purpose: this tower is a ruin.

| Step 14 Set Emberwing on her tower |  |
| --- | --- |

Hind feet tabs go into the top. Front claws slide down over the two front stones.

| Step 15 Add the door, the ivy and the torch |  |
| --- | --- |

| A2 x2 A12 x2 A14 A17 B2 x20 B20 B25 B27 x2 B31 B37 F9 H1 |  |
| --- | --- |

Little square pegs hold each piece.

| Step 16 Scatter the fallen stones |  |
| --- | --- |

| A5 x4 A6 A8 A9 A25 B4 B12 B15 B33 B34 B38 C18 C22 |  |
| --- | --- |

Each pile sits on one peg. Big stone at the bottom.

| Step 17 Raise the banner |  |
| --- | --- |

| A1 x2 A4 D2 |  |
| --- | --- |

The pole goes into the tower top. The flag hooks on.

| Step 18 Build the treasure chest |  |
| --- | --- |

| A13 x2 A26 B2 x2 B5 C8 C14 C17 D4 D5 |  |
| --- | --- |

Gold bands go on last.

| Step 19 Hinge the lid |  |
| --- | --- |

| A19 A24 F7 x2 |  |
| --- | --- |

Push the long dowel through both sides. The lid should swing open.

TEST IT NOW: Open and close the lid. It should not rub the walls.

| Step 20 Push the hinge dowel through |  |
| --- | --- |

| Dowel: 3/16 in dowel, 46 mm |  |
| --- | --- |

Wax it first. Then try the lid.

TEST IT NOW: Open and close the lid. It should not rub the walls.

| Step 21 Pile up the gold |  |
| --- | --- |

| A5 x2 A23 F2 F3 x3 F6 x3 F8 x3 |  |
| --- | --- |

Stack the coins on the little pegs.

You built it!

Help and spare parts: TODO_LUCAS support email