Star Slinger

A starship that slings rubber bands. Seven shots, one at a time.

153 pieces  |  About 5 to 6 h  |  Ages 14+  |  No glue needed

Before you start

|  | Push parts out from the back of the sheet. Do not twist them out. |
| --- | --- |
|  | Every part has a code. B12 is part 12 on sheet B. Codes for show faces are printed next to the part. |
|  | Press tabs straight in, firm but gentle. No glue needed. |
|  | Too tight? Rub the tab on the sandpaper two or three times, then try again. |
|  | Rub wax into every hole that turns. It makes moving parts smooth. |
|  | A check mark means: test it now, before you close it up. |
|  | Small parts. Not for children under 14. Keep spare parts until you finish. |
|  | Wear glasses when you fire. Only aim at the target. Never at faces, people or pets. |

What's in the box

Sheet A: Natural basswood, 78 parts + 8 spares

Sheet B: Natural basswood, 9 parts + 47 spares

Sheet C: Sunset Orange (blaze tip), 38 parts + 2 spares

Sheet D: Haze Blue, 28 parts

12 x #64 rubber bands (3.5 x 1/4 in), ammo

2 x #16 rubber band, trigger return

3 x 3/16 in birch dowel, 15.6 mm

1 x wax stick

1 x sandpaper 240 grit

3 x paper targets (printed)

1 x 3/16 in birch dowel, 63 mm (target hinge)

Parts maps

| Step 1 Lay down the left inner walls |  |
| --- | --- |

| A15 B6 C12 |  |
| --- | --- |

Build the ship lying on its left side.

| Step 2 Stand the ribs in the left walls |  |
| --- | --- |

| A3 A4 A5 A9 A11 A12 A18 A21 A24 C5 C10 |  |
| --- | --- |

Each rib has tabs on both long edges.

| Step 3 Build the star wheel and put it on its axle |  |
| --- | --- |

| A2 x2 B1 x2 D19 Dowel: 3/16 in dowel, 15.6 mm |  |
| --- | --- |

Three discs, two pegs. Wax the axle hole.

TEST IT NOW: Spin the wheel with a finger. It should turn freely and wobble less than 1 mm.

| Step 4 Build the trigger and fit it under the wheel |  |
| --- | --- |

| A2 x2 A7 B2 C21 Dowel: 3/16 in dowel, 15.6 mm Dowel: 3/16 in dowel, 15.6 mm; the #16 band runs from here to the hook on the trigger |  |
| --- | --- |

The two pallets must point at the wheel. Wax the pivot.

TEST IT NOW: Press the trigger slowly: the wheel turns half a tooth. Release: another half tooth.

| Step 5 Close it with the right inner walls |  |
| --- | --- |

| A15 B6 C12 |  |
| --- | --- |

Line up the three dowels and all the rib tabs, then press evenly.

TEST IT NOW: Pull the trigger 8 times. The wheel should step one tooth per pull.

| Step 6 Build the blaze orange nose |  |
| --- | --- |

| C6 C7 C8 x2 C24 C27 C28 |  |
| --- | --- |

This orange box must stay on the ship: it marks it as a toy.

| Step 7 Add the right hull skins |  |
| --- | --- |

| A1 x16 A6 A17 A23 C19 C22 D2 D11 D18 D22 |  |
| --- | --- |

Square pegs lock each skin to the wall.

| Step 8 Turn the ship over and add the left hull skins |  |
| --- | --- |

| A1 x16 A10 A19 A25 C31 C34 D6 D10 D12 D13 |  |
| --- | --- |

Mirror image of the last step.

| Step 9 Add the reactor rings |  |
| --- | --- |

| A1 x4 C32 C33 D20 D21 |  |
| --- | --- |

Rings sit around the wheel axle on both sides.

| Step 10 Fit the cockpit canopy |  |
| --- | --- |

| A2 D9 x2 D23 |  |
| --- | --- |

The canopy tabs go into the deck rib.

| Step 11 Slide in the wings |  |
| --- | --- |

| A1 x2 A2 x2 A14 A16 B4 B5 C1 C2 C4 C17 C18 C20 C23 C25 D15 x2 |  |
| --- | --- |

Each wing has two tabs that go through the skin and the wall.

| Step 12 Raise the tail fin |  |
| --- | --- |

| A2 C13 C14 D17 |  |
| --- | --- |

Its two tabs go into the top rib behind the wheel.

| Step 13 Build the thruster |  |
| --- | --- |

| A8 A22 C15 C16 D3 D4 D8 |  |
| --- | --- |

The peg goes into the tail. Then stack the rings, big ones first.

| Step 14 Add the grip panels |  |
| --- | --- |

| A1 x4 A13 B7 |  |
| --- | --- |

Textured panels on both sides of the grip.

| Step 15 Add the belly fin |  |
| --- | --- |

| A2 C9 C11 D14 |  |
| --- | --- |

Tabs up into the keel, then a stripe on each side and one peg.

| Step 16 Build the launch stand and the target |  |
| --- | --- |

| A1 x2 A2 A20 B3 C3 x2 C26 C29 C30 C35 D1 D5 x2 D7 D16 x2 D24 Dowel: 3/16 in dowel, 63 mm |  |
| --- | --- |

The target hangs on its dowel and swings when hit.

TEST IT NOW: Tap the target plate. It should swing back and settle again.

| Step 17 Drop the ship onto its launch stand |  |
| --- | --- |

Grip between the two boots, barrel in the cradle. No tabs: it lifts straight off.

TEST IT NOW: Lift the ship straight up, then set it back. It should drop in by itself and not wobble.

You built it!

Help and spare parts: TODO_LUCAS support email